Network Time Synchronization System

# 1. Problem Statement

- Build a simplified time-synchronization application using UDP sockets.

- Multiple clients synchronize their clocks with a reference server.

- Run on an emulated network with controlled delay, prioritized sync traffic and congestion.

# 2. Components

- Reference server: h1 (10.0.0.1), timesync_server.py

- Clients: h2 (10.0.0.2), h3 (10.0.0.3), timesync_client.py

- Congestion generator: h4 (10.0.0.4), iperf UDP

- Switch: s1 (Open vSwitch) with built-in OVS controller

- Topology script: topo_sdn.py

- Plotting script: plot_results.py

# 3. Requirement 1: UDP Timestamp Exchange

- Client sends T1 (8-byte double) to server UDP port 12345.

- Server records T2 on receive and T3 before reply.

- Server replies with T1, T2, T3 (24 bytes, network byte order, format !ddd).

- Client records T4 on receive.

- Each client runs 20 probes at 0.5 s intervals with a 2 s timeout.

- Each probe is logged to CSV: Iteration, T1, T2, T3, T4, Delay_ms, Offset_ms.

# 4. Requirement 2: Round-Trip Delay

- Delay = (T4 - T1) - (T3 - T2)

- Server processing time (T3 - T2) is removed from the round trip.

- Reported in milliseconds.

# 5. Requirement 3: Clock Offset Estimation

- Offset = ((T2 - T1) + (T3 - T4)) / 2

- Assumes equal forward and return path delay.

- Reported in milliseconds.

# 6. Requirement 4: Multiple Clients

- Server handles requests from any client address in one receive loop.

- Replies are sent to the source address of each request.

- Clients h2 and h3 both synchronize with h1.

- h2 (10 ms link) and h3 (15 ms link) give different delays, as expected.

# 7. Requirement 5: Prioritize Synchronization Traffic

- Client marks sync packets with IP_TOS 0xB8 (DSCP 46, Expedited Forwarding).

- OpenFlow rule on s1: priority 65535, UDP, tp_dst=12345, actions=normal.

- Default rule on s1: priority 100, IP, actions=normal.

- Rules installed with ovs-ofctl after the network starts.

# 8. Requirement 6: Controlled Network Delay

- Mininet TCLink with fixed delay and 10 Mbps bandwidth per host link.

- h1: 5 ms. h2: 10 ms. h3: 15 ms. h4: 2 ms.

- Expected RTT for h2 = 2 x (10 + 5) = 30 ms.

- Expected RTT for h3 = 2 x (15 + 5) = 40 ms.

- Measured RTT for h2 = 30.79 ms (probes 2-20).

- Measured RTT for h3 = 40.54 ms (probes 2-20).

# 9. Requirement 7: Synchronization Error

- All Mininet hosts share one system clock, so the true offset is 0 ms.

- The measured offset is therefore the estimation error.

- h2, normal, probes 2-20: mean offset 0.148 ms.

- h3, normal, probes 2-20: mean absolute offset 0.069 ms.

- h2, congested: mean offset 0.28 ms, max 0.67 ms.

- Probe 1 outlier: h2 16.11 ms, h3 21.31 ms (start-up cost such as ARP).

# 10. Requirement 8: Performance under Congestion

- Congestion source: h4 sends UDP iperf at 10 Mbps (iperf -c 10.0.0.2 -u -b 10M -t 30).

- Runs recorded: sync_normal.csv (h2), sync_h3_normal.csv (h3), sync_congested.csv (h2).

- Normal h2: mean delay 32.41 ms, max 63.33 ms (30.79 ms mean without probe 1).

- Normal h3: mean delay 42.69 ms, max 83.48 ms (40.54 ms mean without probe 1).

- Congested h2: mean delay 31.09 ms, max 31.87 ms, 0 timeouts.

- Congested probes 1-11: delay 31.2-31.9 ms, offset 0.36-0.67 ms.

- Congested probes 12-20: delay about 30.6 ms, offset under 0.1 ms.

Figure 1: Round-trip delay and offset, normal vs congested (h2).

# 11. Scope Notes

- No congested run without the priority rule was recorded, so no with/without comparison is reported.

- The priority rule uses actions=normal; no separate queue is configured.

- Clients estimate the offset; they do not adjust a clock.

- Timeouts are not logged, so packet loss is not tabulated.

- h2 and h3 runs were recorded separately.

# 12. Conclusion

- UDP timestamp exchange, delay calculation and offset estimation work as designed.

- Two clients synchronize with one server over links with controlled delay.

- Measured RTTs match the configured delays within about 1 ms.

- Offset error stays below 0.7 ms after the first probe in all runs.

- With cross-traffic and the priority rule in place, delay rose by about 0.5 ms.

# Appendix: How to Run

sudo python3 topo_sdn.py

mininet> h1 python3 timesync_server.py &

mininet> h2 python3 timesync_client.py 10.0.0.1 sync_normal.csv

mininet> h3 python3 timesync_client.py 10.0.0.1 sync_h3_normal.csv

mininet> h4 iperf -c 10.0.0.2 -u -b 10M -t 30 &

mininet> h2 python3 timesync_client.py 10.0.0.1 sync_congested.csv

python3 plot_results.py sync_normal.csv sync_congested.csv